# TRANSMISSION LINES & WAVEGUIDES – ECA1501

## EXPERIMENT – 2

| Name | B.Manikanta reddy | Reg. No. | 192412520 |
| --- | --- | --- | --- |
| Date | 12/10/2026 | Experiment No. | 2 |

Analysis of SWR (Standing wave ratio) Transmission lines by varying a) Load Impedance

b) Characteristic Impedance

### AIM

### To analyze the SWR of a transmission line for a fixed characteristic impedance (Zo) of 50

### ohms, as the load impedance (ZL) is varied across the following values: 25 ohms, 50 ohms, 75

### ohms, and 100 ohms. To verify how the SWR changes with each value of ZL and when ZL =Zo.

### SOFTWAREREQUIRED:

### SCILABVersion6.1.1

### PROCEDURE:

### ●Open SCILAB software

### ● Open file-new

### ● Put the input values

### ● Type the program

### ● Execute the program

### ● End the program

PROGRAM:

clc;clear;close;

Zo=200;ZR=650-(%i*475);

K=(ZR-Zo)/(ZR+Zo);

ampK=sqrt((real(K)^2)+(imag(K)^2));S=(1+ampK)/(1-ampK);

printf("(c)Standing wave ratio = %f",round(S*1000)/1000);

### TEST CASE 1

### Analyze the SWR of a transmission line for a fixed characteristic impedance (Zo) of 50

### ohms, as the load impedance (ZL) is varied across 650 ohms to 850 ohms. Verify how the

### SWR changes with each value ofZL. When ZL=Z0, the SWR should equal 1 (indicating no

### reflection).

OUTPUT TABLE 1

Zo=50 ohms Variable: ZR

| ZR (Ohms) | Simulation Results (SWR) | Theoretical Results (SWR) |
| --- | --- | --- |
| 50 | 1.000 | 1.000 |
| 100 | 2.000 | 2.000 |
| 150 | 3.000 | 3.000 |
| 200 | 4.000 | 4.000 |
| 250 | 5.000 | 5.000 |
| 300 | 6.000 | 6.000 |

ZR=50/40ohmsVariable: Zo

| Z0 (Ohms) | Simulation Results (SWR) | Theoretical Results (SWR) |
| --- | --- | --- |
| 50 | 2.145 | 2.145 |
| 100 | 2.921 | 2.921 |
| 150 | 4.108 | 4.108 |
| 200 | 5.361 | 5.361 |
| 250 | 6.637 | 6.637 |
| 300 | 7.924 | 7.924 |

RESULT:

The SWR of a transmission line for a fixed characteristic impedance (Zo) of 50 ohms, is

analyzed for various values of load impedance ZL. SWR is also examined for the condition ZL

=Zo.

### TEST CASE 2

- Analyze the SWR for a fixed load impedance (ZL) of 50 ohms while varying the

- characteristic impedance (Z0) of the transmission line to 250 ohms to 400 ohms. Verify how

- the SWR changes with each value Z0 including Z0=ZL. Variable Resistance (R): L = 3.5 mH/km, C = 8 µF/km, G = 4 µmho/km, f = 2.5 kHz

OUTPUT:

ZR=50/_40o ohms Variable: Zo

| Z0 (Ohms) | Simulation Results (SWR) | Theoretical Results (SWR) |
| --- | --- | --- |
| 50 | 2.145 | 2.145 |
| 100 | 2.921 | 2.921 |
| 150 | 4.108 | 4.108 |
| 200 | 5.361 | 5.361 |
| 250 | 6.637 | 6.637 |
| 300 | 7.924 | 7.924 |

RESULT:

SWR for a fixed load impedance (ZL) of 50 ohms is analyzed while varying the characteristic

impedance (Z0) of the transmission line to 250 ohms to 400 ohms. Impact of the condition Z0=ZL

on SWR is also examined for various values of Z0 .